CABINET FOR HEALTH AND FAMILY SERVICES

Department for Public Health

Division of Public Health Protection and Safety

(Amended After Comments)

902 KAR 10:125. Kentucky public swimming and bathing facility safety requirements.

RELATES TO: KRS 211.015, 211.205

STATUTORY AUTHORITY: KRS 194A.050, 211.180, 2024 Ky Acts ch. 116

NECESSITY, FUNCTION, AND CONFORMITY: KRS 194A.050(1) authorizes the secretary of the Cabinet for Health and Family Services to promulgate administrative regulations necessary to protect, develop, and maintain the health, personal dignity, integrity, and sufficiency of Kentucky citizens and to operate programs and fulfill the responsibilities vested in the cabinet. KRS 211.180 authorizes the cabinet to adopt administrative regulations relating to public facilities, and their operation and maintenance in a safe and sanitary manner to protect public health and prevent health hazards. This administrative regulation establishes uniform safety standards for public swimming pools and bathing facilities.

Section 1. Definitions.

(1) "Bather" means a person using a public swimming and bathing facility.

(2) "Bather load" means all bathers within the public swimming and bathing facility enclosure.

(3) "Cabinet" is defined by KRS 211.015(1)(a).

(4) "Class A" is defined by 2024 Ky Acts ch. 116 Section 1(1)(a).

(5) "Class B" is defined by 2024 Ky Acts ch. 116 Section 1(1)(b).

(6) "Play feature" means a structure or feature that is added to a pool for the purpose of entertainment.

(7) "Public swimming and bathing facility" or "facility" means a natural or artificial body or basin of water that is modified, improved, constructed, or installed for the purpose of swimming or bathing, except for a pool at a private single-family residence intended only for the use of the occupant[owner] and guests.

(8) "Public swimming and bathing facility enclosure" means an enclosure that surrounds and secures the public swimming and bathing facility that includes decking and pool.

(9) "Readily accessible" means direct access without the necessity of removing any panel, door, or similar obstruction.

(10) "Splash pad" means a public swimming and bathing facility that:

(a) Has aquatic play features that spray or drop water for the purpose of wetting people;

(b) Is designed so that there is no accumulation or ponding of water on the ground;

(c) Includes both recirculating and non-recirculating water systems; and

(d) Includes splash pads operated by local governments as defined in KRS 211.205.

(11) "Third-party compliance specialist" means a person who is a representative of an incorporated organization that provides lifeguard training and pool safety analysis and is not a representative of state and local governments nor is an associate of the establishment seeking use of an alternative lifeguard plan.

Section 2. Lifeguards.

(1) Class A and Class B pools that meet the criteria specified in 2024 Ky Acts ch. 116 Section 1(2) shall have lifeguards on duty at a rate of one (1) per 100 bathers.

(2) All Class A pools over 2,000 square feet shall have a minimum of one (1) lifeguard on duty at all times the pool is open to bathers.

(3) Additional lifeguards shall be provided at a rate of:

(a) One per 2,000 square feet or major fraction more than half thereof according to the following table:

| Water Surface Square Footage | Number of Lifeguards Required |
| --- | --- |
| 2,000-3,000 | 1 lifeguard required |
| 3,001-5,000 | 2 lifeguards required |
| 5,001-7,000 | 3 lifeguards required |
| 7,001-9,000 | 4 lifeguards required |
| 9,001-11,000 | 5 lifeguards required |
| 11,001 and above | 6 lifeguards required plus one additional lifeguard for each additional 2,000 square feet over 11,000 square feet; |

or

(b) One (1) per 100 bathers according to the following table:

| Number of Bathers | Number of Lifeguards Required |
| --- | --- |
| 1-100 | 1 lifeguard required |
| 101-200 | 2 lifeguards required |
| 201-300 | 3 lifeguards required |
| 301-400 | 4 lifeguards required |
| 401-500 | 5 lifeguards required |
| 501-600 | 6 lifeguards required |
| 601 and above | 7 lifeguards required plus one additional lifeguard for each additional 100 bathers. |

(4) All Class A and B pools using number of bathers for the lifeguard application rate shall:

(a) Establish a method for tracking bathers entering the facility;

(b) Continuously monitor fluctuating bather load and staff lifeguards accordingly; and

(c) Upon inspection, provide accurate bather load logs to the cabinet.

(5) In accordance with 2024 Ky Acts ch. 116 Section 1(4), a swimming coach or instructor may count as a required lifeguard. The swimming coach or instructor shall comply with the requirements of subsection (10)(a) of this section when acting as a required lifeguard.

(6) All facilities that are not required to provide lifeguards shall post and enforce the following rules at all entrance points: "No Lifeguard on Duty" and "No person may enter the facility area alone or swim alone."

(7) In accordance with 2024 Ky Acts ch. 116 Section 1(2), at all times when a lifeguard is not on duty, features such as induced waves, slides, diving boards, platforms, climbing walls, or other similar features shall not be used. These features shall be either roped off or otherwise blocked for usage, and signage shall clearly indicate that the features may not be used.

(8) Splash pads shall not be required to provide lifeguards.

(9) A facility may submit an alternative lifeguard staffing plan that:

(a) Has been certified by an independent third-party compliance specialist who witnessed the initial testing to ensure the plan is sufficient to protect patrons;

(b) Designates the number of lifeguards necessary to ensure each lifeguard is capable of viewing the entire area of the assigned zone of patron surveillance;

(c) Ensures the lifeguard is able to reach the farthest extent of the assigned zone of patron surveillance within twenty (20) seconds; and

(d) Includes the following:

1. A description of study methods and calculations used to determine lifeguard zones and placement;

2. Methods and frequency of testing that will be performed to ensure the plan protects patrons;

3. A complete layout of the pool surface diagrammed with lifeguard placement and assigned zone of surveillance; and

4. A minimum baseline of lifeguards required for safe operation;

(e) The alternative lifeguard staffing plan shall be:

1. On file with the Public Safety Branch within the Department for Public Health;

2. Submitted to the local health department of jurisdiction; and

3. Resubmitted if there is a change in:

a. The shape or size of the swimming pool;

b. The surrounding areas that would obstruct the lifeguard's view of the bottom of the pool; or

c. Ownership of the facility.

(10) Lifeguards shall comply with the following:

(a) Lifeguards, including coaches or instructors serving as lifeguards as described in 2024 Ky Acts ch. 116 Section (1)(4), shall have a current lifesaving certificate. Current training as a lifesaver or water safety instructor shall be an American Red Cross or equivalent training and include an in-water training component. The certificate of competency shall be onsite and available for inspection upon request;

(b) Lifeguards shall be dressed in swimming attire; and

(c) Lifeguards assigned to the supervision of the facility shall not be subject to duties that would:

1. Distract their attention from proper observation of persons in the facility area; or

2. Prevent immediate assistance to persons in distress in the water.

Section 3. Safety Equipment.

(1) One (1) unit consisting of the following lifesaving equipment shall be provided for 2,000 square feet of water surface area and an additional unit for each additional 2,000 square feet or major fraction more than half thereof:

(a) A U.S. Coast Guard approved ring buoy no more than twenty (20) inches in diameter with a three-sixteenths (3/16) inch rope attached that measures one and one-half (1 1/2) times the maximum pool width;

(b) Rescue tubes may be used when lifeguards are present;

(c) A shepherd's hook securely attached to a one (1) piece pole not less than twelve (12) feet in length; and

(2) One (1) backboard with head immobilizer and at least three (3) straps for back and neck injuries shall be provided per facility.

(3) All facilities shall be equipped with a minimum of one (1) standard twenty-four (24) unit first aid kit or its equivalent that is kept filled and ready for use. Additional units shall be provided for each additional 2,000 square feet of facility area or major fraction more than half thereof.

(4) Spas with less than 144 square feet of water surface areas shall be exempt from the requirements of subsection (1) and (2) of this section.

(5) Splash pads shall be exempt from the requirements of subsection (1) through (3) of this section.

(6) Lifesaving equipment shall be mounted in conspicuous places at lifeguard chairs or other readily accessible locations. Its function shall be plainly marked, and this equipment shall be kept in repair and ready condition. Bathers or other persons shall not be permitted to tamper with, use for any purpose other than its intended use, or remove this equipment from its established location.

(7) The hydrojet auxiliary air or water pump for a spa shall be controlled by an on-off switch with a fifteen (15) minute timer located and labeled at least five (5) feet away from the spa.

(8) All facilities shall provide an emergency automatic pump shut off readily accessible by facility staff.

Section 4. Emergency Telephones.

(1) All facilities shall have a non-pay landline or Voice over Internet Protocol (VoIP) telephone, continuously connected to a power source and operational at all times, capable of direct dialing 911 without going through a switchboard, and located on the deck in a conspicuous, readily accessible location. A cordless telephone shall be prohibited. Instructions for dialing shall be posted if necessary.

(2) All facilities utilizing VoIP telephones shall only use fixed VoIP services. Non-fixed VoIP services shall be prohibited.

(3) The address of the facility and the telephone number of the police department, fire department, emergency medical service, or a hospital shall be posted in a conspicuous place near the telephone.

Section 5. Reporting Requirements. All drownings, near drownings, and injuries requiring hospitalization shall be immediately reported to the local health department and the Department for Public Health on form "DFS-354, Kentucky Public Swimming and Bathing Facilities Drowning and Injury Report".

Section 6. Existing Facilities and Equipment. Existing facilities and equipment being used prior to the effective date of this administrative regulation that do not fully meet the design, construction, and materials requirements of this administrative regulation, may continue to be used if the facilities and equipment:

(1) Are in good repair;

(2) Are capable of being maintained in a sanitary condition; and

(3) Create no health or safety hazard.

Section 7. Variances.

(1) A facility submitting an alternative lifeguard staffing plan pursuant to Section 2(7) of this administrative regulation may submit a request for a variance to the safety equipment requirements of this section to the Environmental Management Branch in the Department for Public Health.

(2) The variance requested shall not affect the safe and healthful operation of the facility.

(3) Before granting a variance, the cabinet shall require adequate proof from the applicant that the requested variance will comply with the basic intent of this section and that no safety or health hazard would be created if the variance is granted.

Section 8. Incorporated by Reference.

(1) The "DFS-354, Kentucky Public Swimming and Bathing Facilities Drowning and Injury Report", 3/2024, is incorporated by reference.

(2) This material may be inspected, copied, or obtained, subject to applicable copyright law, at the Division of Public Health Protection and Safety, Department for Public Health, 275 East Main Street, Frankfort, Kentucky 40621, Monday through Friday, 8 a.m. to 4:30 p.m. and online at https://chfs.ky.gov/agencies/dph/dphps/emb/Pages/pools.aspx.

STEVEN J. STACK, MD, MBA, Commissioner

ERIC C. FRIEDLANDER, Secretary

APPROVED BY AGENCY: November 12, 2024

FILED WITH LRC: November 13, 2024 at 3:30 p.m.

CONTACT PERSON: Krista Quarles, Policy Analyst, Office of Legislative and Regulatory Affairs, 275 East Main Street 5 W-A, Frankfort, Kentucky 40621; phone 502-564-7476; fax: 502-564-7091; email CHFSregs@ky.gov.

REGULATORY IMPACT ANALYSIS AND TIERING STATEMENT

Contact Person: Julie Brooks or Krista Quarles

(1) Provide a brief summary of:

(a) What this administrative regulation does:

This administrative regulation establishes the safety requirements for public swimming and bathing facilities. The amended after comments version clarifies the applicability of the requirements for public swimming pools, and updates language regarding location of the telephone for consistency.

(b) The necessity of this administrative regulation:

This administrative regulation is necessary to ensure all public swimming and bathing facilities operate in a safe and sanitary manner to reduce the incidence of recreational water related illnesses and injuries. According to the Centers for Disease Control and Prevention (CDC): "Water-based physical activity, such as swimming, improves physical and mental health throughout life; however, it can put people at risk for recreational water–associated illness and injury. A recreational water illness outbreak is the occurrence of similar illnesses in 2 or more persons, epidemiologically linked by location and time of exposure to recreational water. For 1978–2012, 879 recreational water illness outbreaks have been reported CDC, and the number reported annually has increased significantly in recent years, especially the number of outbreaks associated with treated recreational water venues (e.g., pools and hot tubs/spas) reported annually". Fatal drownings are the leading cause of injury death for US children ages 1 to 4, and the third leading cause of unintentional injury death for US youth ages 5 to 19 (American Academy of Pediatrics). Not all drownings are considered "fatal" drownings; non-fatal drownings can have long term physical and mental health consequences. According to a 1998 report by the CDC on lifeguard effectiveness: "Most drownings are preventable through a variety of strategies, one of which is to provide lifeguards in public areas where people are known to swim and to encourage people to swim in those protected areas. Some estimates indicate that the chance of drowning at a beach protected by lifeguards can be less than one in 18 million. There is no doubt that trained, professional lifeguards have had a positive effect on drowning prevention in the United States."

(c) How this administrative regulation conforms to the content of the authorizing statutes:

KRS 194A.050(1) authorizes the secretary of the Cabinet for Health and Family Services to promulgate administrative regulations necessary to protect, develop, and maintain the health, personal dignity, integrity, and sufficiency of Kentucky citizens and to operate programs and fulfill the responsibilities vested in the cabinet. KRS 211.180 authorizes the cabinet to adopt administrative regulations relating to public facilities, public and semipublic recreational areas, and their operation and maintenance in a safe and sanitary manner to protect public health and prevent health hazards.

(d) How this administrative regulation currently assists or will assist in the effective administration of the statutes:

This administrative regulation will ensure public swimming and bathing facilities operate in a safe and sanitary manner to protect the public health.

(2) If this is an amendment to an existing administrative regulation, provide a brief summary of:

(a) How the amendment will change this existing administrative regulation:

This is a new administrative regulation.

(b) The necessity of the amendment to this administrative regulation:

This is a new administrative regulation.

(c) How the amendment conforms to the content of the authorizing statutes:

This is a new administrative regulation.

(d) How the amendment will assist in the effective administration of the statutes:

This is a new administrative regulation.

(3) List the type and number of individuals, businesses, organizations, or state and local governments affected by this administrative regulation:

There are currently 2,252 public swimming pools, bathing facilities, and spas regulated by the department.

(4) Provide an analysis of how the entities identified in question (3) will be impacted by either the implementation of this administrative regulation, if new, or by the change, if it is an amendment, including:

(a) List the actions that each of the regulated entities identified in question (3) will have to take to comply with this administrative regulation or amendment:

All facility owners and operators will need to be aware of the safety requirements, including staffing requirements, and ensure their facilities are in compliance.

(b) In complying with this administrative regulation or amendment, how much will it cost each of the entities identified in question (3):

There may be minimal cost to facilities to comply with the safety requirements of this administrative regulation.

(c) As a result of compliance, what benefits will accrue to the entities identified in question (3):

Facilities operating in compliance with this administrative regulation will protect the health and safety of those who swim at the facility.

(5) Provide an estimate of how much it will cost the administrative body to implement this administrative regulation:

(a) Initially:

This is an ongoing program, there are no initial cost.

(b) On a continuing basis:

The cost to the Department for Public Health associated with administering the public swimming pool permitting and inspection program is $2,376,885. This is a combined state and local health department total.

(6) What is the source of the funding to be used for the implementation and enforcement of this administrative regulation:

State general fund dollars, permit and inspection fees are the sources of funding for this administrative regulation.

(7) Provide an assessment of whether an increase in fees or funding will be necessary to implement this administrative regulation, if new, or by the change if it is an amendment:

An increase in fees or funding is not needed to implement this administrative regulation.

(8) State whether or not this administrative regulation establishes any fees or directly or indirectly increases any fees:

There are no fees established in this administrative regulation.

(9) TIERING: Is tiering applied?

Tiering is not applied. While some Class A and Class B pools may not be required to provide lifeguards, the required safety rules apply equally to all regulated swimming and bathing facilities.

FISCAL IMPACT STATEMENT

(1) Identify each state statute, federal statute, or federal regulation that requires or authorizes the action taken by the administrative regulation.

KRS 194A.050 and 211.180.

(2) Identify the promulgating agency and any other affected state units, parts, or divisions:

The Kentucky Department for Public Health, Division of Public Health Protection and Safety is the promulgating agency. This administrative regulation also affects Kentucky state parks that operate public swimming and bathing facilities.

(a) Estimate the following for the first year:

Expenditures: The cost to the Department for Public Health associated with administering the permitting and inspection program is $2,376,885. This is a combined total for state and local health departments. Expenditures for Kentucky state parks include the cost associated with maintaining their facilities in compliance with this administrative regulation and associated staffing cost.

Revenues: This administrative regulation does not generate revenue.

Cost Savings: This administrative regulation does not result in cost savings.

(b) How will expenditures, revenues, or cost savings differ in subsequent years?

(3) Identify affected local entities (for example: cities, counties, fire departments, school districts):

This administrative regulation affects local health departments who inspect the permitted facilities for compliance with the safety requirements of this administrative regulation. This administrative regulation also affects local governments that have public swimming and bathing facilities.

(a) Estimate the following for the first year:

Expenditures: for the Department for Public Health may be impacted by changes in salary, fringe benefits and travel cost for state and local health department employees. These changes cannot be determined at this time. Expenditures for Kentucky state parks will be impacted by changes in maintenance cost and associated staffing cost. These totals cannot be determined at this time.

Revenues: This administrative regulation does not generate revenue.

Cost Savings: This administrative regulation does not result in cost savings.

(b) How will expenditures, revenues, or cost savings differ in subsequent years?

Expenditures for local health departments may be impacted by changes in salary, fringe benefits and travel cost for state and local health department employees. These changes cannot be determined at this time. Expenditures to Kentucky state parks may be impacted by changes in the costs associated with facility maintenance as well as changes in associated staff cost.

(4) Identify additional regulated entities not listed in questions (2) or (3):

Additional regulated entities include hotels, recreational facilities, such as water parks, health facilities and athletic clubs, schools including colleges and universities, swim clubs and country clubs, youth camps, and any other entity that provides a public swimming and bathing facility.

(a) Estimate the following for the first year:

Expenditures: The costs associated with administering the permitting and inspection program is $2,376,885. This is a combined total for state and local health departments. Expenditures for local governments will include the cost associated with maintaining the public swimming and bathing facility and associated staffing costs.

Revenues: This administrative regulation does not generate revenue.

Cost Savings: This administrative regulation does not result in cost savings.

(b) How will expenditures, revenues, or cost savings differ in subsequent years?

Expenditures to the additional regulated entities may be impacted by changes in the costs associated with facility maintenance as well as changes in associated staff cost.

(5) Provide a narrative to explain the:

(a) Fiscal impact of this administrative regulation:

This administrative regulation does not add any additional requirements to the regulated entities. Permitted facilities may incur costs associated with facility maintenance as well as associated staffing cost. Those figures cannot be determined at this time.

(b) Methodology and resources used to determine the fiscal impact:

This administrative regulation does not generate revenue. Expenditures for each facility will be dependent on the size of the facility and the facility staffing pattern. A facility that employs lifeguards will incur the cost associated with staffing the facility. All facilities will incur cost associated with maintaining the facility for compliance with this administrative regulation.

(6) Explain:

(a) Whether this administrative regulation will have an overall negative or adverse major economic impact to the entities identified in questions (2) - (4). ($500,000 or more, in aggregate)

This administrative regulation will not have an overall negative or adverse major economic impact.

(b) The methodology and resources used to reach this conclusion:

The costs associated with this administrative regulation are shared costs between local health departments and the cabinet. This administrative regulation does not generate revenue. The administrative costs to local health departments and the cabinet are approximately $150 per hour for the environmental health inspectors of swimming and bathing facilities. This cost includes the salary of the inspector, Kentucky Employee Retirement System (KERS) contributions, Federal Insurance Contributions Act (FICA) contributions, and health and life insurance cost. The total costs across all local health departments and the state are approximately $2,376,885 per year.