# AN ACT relating to plastic containers.

Be it enacted by the General Assembly of the Commonwealth of Kentucky:

# Section 1.   KRS 224.50-585 is amended to read as follows:

## (1)	As used in this section:

### (a)	"Rigid plastic container" means any formed or molded article comprised predominantly of plastic resin and having a relatively inflexible finite shape or form intended primarily as a single service container with a capacity of eight (8) ounces or more and less than five (5) gallons;

### (b)	"Rigid plastic bottle" means any rigid plastic container with a neck that is smaller than the container body with a capacity of sixteen (16) ounces or more and less than five (5) gallons; and

### (c)	"Label" means a code label described in this section molded into the bottom of the plastic product.

## (2)	All rigid plastic bottles and rigid plastic containers sold in Kentucky on and after January 1, 1992, shall be labeled with a code which indicates the resin used to produce the rigid plastic bottle or rigid plastic container. The code shall consist of a number placed inside a triangle and letters placed below the triangle. The triangle shall be equilateral, formed by three (3) arrows with the apex of each point of the triangle at the midpoint of each arrow, rounded with a short radius. The pointer or arrowhead of each arrow shall be at the midpoint of each side of the triangle with a short gap separating the pointer from the base of the adjacent arrow. The triangle, formed by the three (3) arrows curved at their midpoints, shall depict a clockwise path around the code number. The numbers and letters used shall be as follows:

1 = PETE (polyethylene terephthalate)

2 = HDPE (high density polyethylene)

3 = V (vinyl)

4 = LDPE (low density polyethylene)

5 = PP (polypropylene)

6 = PS (polystyrene)

7 = OTHER (represents all other resins, including layered plastics

of a combination of materials).

## (3)	A plastic bottle or rigid plastic container having a plastic product label, base cup, layer of resin, or other plastic component made of material different from that constituting the rest of the bottle or container shall be labeled with:

### (a)	The code for the material constituting the bottle or container if the material constituting the product label, base cup, layer of resin, or other plastic component is the same predominant resin as that in the bottle or container so as to be compatible for purposes of recycling; and

### (b)	The word "OTHER" and the number "7" if the material constituting the product label, base cup, layer of resin, or other plastic component is not compatible for the purposes of recycling with the material constituting the bottle or container.