# AN ACT relating to gravel operations.

Be it enacted by the General Assembly of the Commonwealth of Kentucky:

# âSECTION 1.   A NEW SECTION OF KRS CHAPTER 350 IS CREATED TO READ AS FOLLOWS:

## (1)	The cabinet shall promulgate administrative regulations providing for a "limited gravel permit" for gravel extraction conducted for agricultural or personal use where:

### (a)	The area from which the gravel is extracted does not exceed one (1) acre;

### (b)	The extraction is done by the owner of the property;

### (c)	The excavated material is not washed, screened, or graded prior to use; and

### (d)	Any stockpiling of the excavated material occurs in the pit area in order to prevent runoff to surface waters from stockpiled material.

## (2)	The limited gravel permit application shall be submitted on forms provided by the cabinet and shall include:

### (a)	The location of the proposed operation;

### (b)	A statement that the applicant owns both the surface and mineral rights to the property; and

### (c)	Documentation demonstrating compliance with the following requirements:

#### 1.	The total area on which extraction will occur shall not exceed one (1) acre, and the total depth of the excavation shall not exceed an average of twelve (12) feet;

#### 2.	No portion of the excavation or loading area shall be within one hundred (100) feet of a property line, public road, cemetery, ephemeral or intermittent or perennial stream or other watercourse, nor within one hundred (100) feet of an occupied dwelling, unless a closer distance is authorized in writing by the dwelling owner;

#### 3.	No blasting or fracturing of rock strata by vibrational equipment shall be conducted in connection with the gravel extraction;

#### 4.	The proposed excavation shall not affect wetlands or endangered or threatened species and their habitats, and shall not otherwise pose a threat to the public safety or the environment; and

#### 5.	A description of best management practices that shall be employed for the protection of surface and groundwater quality and to minimize dust generated from the extraction and loading activity.

## (3)	A limited gravel permit shall not be transferable and only one (1) such permit shall be authorized per property at any one (1) time. The permittee applicant may not hold, own, or control a currently active limited gravel operation or other noncoal permit issued pursuant to 405 KAR Chapter 5 and no limited gravel permit shall be issued on a property where an existing gravel excavation has not been reclaimed.

## (4)	The activity authorized by the permit shall be conducted by the applicant and not by a third party. The gravel extracted shall not be resold.

## (5)	The cabinet may suspend or revoke a limited gravel permit for any failure to comply with the terms and conditions of the permit or if the excavation activity creates nuisance conditions or poses a threat to public safety or the environment.

## (6)	If the total area disturbed for extraction under a limited gravel permit exceeds one (1) acre, then the entire excavation shall become subject to all applicable requirements of KRS Chapter 350 and 405 KAR Chapter 5, including assessment of penalties for noncompliance with this section.